/* ESP32 HTTP Client for Server Communication

- Sends sensor data to server every 0.1 second

- Polls server for commands every 0.1 second

- pH on ADC34, LDR on ADC35, DHT11 on GPIO25

- Relays on GPIOs 5, 18, 19 (active LOW)

- WiFi: SSID "Projects", password "12345678@"

*/

#include <WiFi.h>

#include <HTTPClient.h>

#include <ArduinoJson.h>

#include "DHT.h"

#include <WiFiClientSecure.h>

#define PH_SENSOR_PIN 34   // ADC pin (pH)

#define LDR_PIN 35         // ADC pin (LDR)

#define DHTPIN 25          // DHT11 data pin

#define DHTTYPE DHT11

// Relay pins (GPIO numbers)

const int RELAY_PIN_1 = 5;   // Relay1 controlled by LDR

const int RELAY_PIN_2 = 18;  // Relay2 manual water pump

const int RELAY_PIN_3 = 19;  // Relay3 manual fan

// WiFi credentials

const char* ssid = "Smart_agri";

const char* password = "12345678";

// Server configuration

const char* server_url = "https://agri.msiproject.space";

String sensor_endpoint = String(server_url) + "/api/sensor-data";

String commands_endpoint = String(server_url) + "/api/commands";

DHT dht(DHTPIN, DHTTYPE);

// SSL Configuration for HTTPS

WiFiClientSecure client;

HTTPClient http;

// pH calibration

float calibration_value = 21.34 + 1.5;

// pH smoothing buffers

unsigned long avgval;

int buffer_arr[10], temp;

float ph_act;

// Sensor storage

int ldr_raw = 0;

float ldr_percent = 0.0;

float temperature = NAN;

float humidity = NAN;

const float ADC_MAX = 4095.0; // 12-bit ADC

const float VREF = 3.3;

// Relay states (manual only)

bool relayState1 = false;

bool relayState2 = false;

bool relayState3 = false;

// Timing variables

unsigned long lastSensorMillis = 0;

unsigned long lastDataSendMillis = 0;

unsigned long lastCommandCheckMillis = 0;

const unsigned long SENSOR_INTERVAL = 100;       // 0.1 second (fast local sensing)

const unsigned long DATA_SEND_INTERVAL = 1000;   // 1.0 second (reduce blocking I/O)

const unsigned long COMMAND_CHECK_INTERVAL = 150; // 0.15 second (faster command polling)

// Function declarations

void readSensors();

void sendDataToServer();

void checkForCommands();

void processCommand(JsonObject command);

void applyRelayOutput(int pin, bool state);

float getLastPHVoltage();

void setup() {

Serial.begin(115200);

delay(10);

Serial.println("\nStarting ESP32 HTTP Client...");

// ADC setup

analogReadResolution(12);

analogSetPinAttenuation(PH_SENSOR_PIN, ADC_11db);

analogSetPinAttenuation(LDR_PIN, ADC_11db);

// DHT

dht.begin();

// SSL Configuration for HTTPS

client.setInsecure(); // Skip certificate verification for now

// Alternative: client.setCACert(ca_cert); // Use proper certificate

// Note: setInsecure() bypasses SSL certificate verification

// For production, use proper certificate validation

client.setTimeout(1500); // keep network reads snappy

// Reuse HTTP connection where possible

http.setReuse(true);

http.setTimeout(1000); // fail fast on slow server

// Relay pins - set as OUTPUT and set HIGH (inactive because relays are active LOW)

pinMode(RELAY_PIN_1, OUTPUT);

pinMode(RELAY_PIN_2, OUTPUT);

pinMode(RELAY_PIN_3, OUTPUT);

digitalWrite(RELAY_PIN_1, HIGH);

digitalWrite(RELAY_PIN_2, HIGH);

digitalWrite(RELAY_PIN_3, HIGH);

// WiFi connection

WiFi.mode(WIFI_STA);

WiFi.setSleep(false); // reduce WiFi latency

WiFi.begin(ssid, password);

Serial.print("Connecting to WiFi ");

Serial.print(ssid);

unsigned long start = millis();

while (WiFi.status() != WL_CONNECTED && millis() - start < 15000) {

delay(50);

Serial.print(".");

}

if (WiFi.status() == WL_CONNECTED) {

Serial.println();

Serial.print("Connected! IP: ");

Serial.println(WiFi.localIP());

Serial.print("Server URL: ");

Serial.println(server_url);

} else {

Serial.println();

Serial.println("WiFi connection failed!");

ESP.restart();

}

Serial.println("ESP32 HTTP Client Ready");

delay(100);

}

void loop() {

unsigned long now = millis();

// Ensure physical relays match stored states (manual only)

applyRelayOutput(RELAY_PIN_1, relayState1);

applyRelayOutput(RELAY_PIN_2, relayState2);

applyRelayOutput(RELAY_PIN_3, relayState3);

// Read sensors every 0.1 second

if (now - lastSensorMillis >= SENSOR_INTERVAL) {

lastSensorMillis = now;

readSensors();

}

// Send data to server at reduced rate to avoid blocking loop

if (now - lastDataSendMillis >= DATA_SEND_INTERVAL) {

lastDataSendMillis = now;

if (WiFi.status() == WL_CONNECTED) {

sendDataToServer();

}

}

// Check for commands frequently but not excessively

if (now - lastCommandCheckMillis >= COMMAND_CHECK_INTERVAL) {

lastCommandCheckMillis = now;

if (WiFi.status() == WL_CONNECTED) {

checkForCommands();

}

}

// Reconnect WiFi if disconnected

if (WiFi.status() != WL_CONNECTED) {

Serial.println("WiFi disconnected, reconnecting...");

WiFi.reconnect();

delay(500);

}

}

void readSensors() {

// --- pH sampling (10 samples, sort, average middle 6) ---

for (int i = 0; i < 10; i++) {

buffer_arr[i] = analogRead(PH_SENSOR_PIN);

delay(1);

}

// Sort array

for (int i = 0; i < 9; i++) {

for (int j = i + 1; j < 10; j++) {

if (buffer_arr[i] > buffer_arr[j]) {

temp = buffer_arr[i];

buffer_arr[i] = buffer_arr[j];

buffer_arr[j] = temp;

}

}

}

// Average middle 6 values

avgval = 0;

for (int i = 2; i < 8; i++) avgval += buffer_arr[i];

float avgRaw = (float)avgval / 6.0;

float volt = avgRaw * VREF / ADC_MAX;

ph_act = -5.70 * volt + calibration_value;

// --- LDR ---

ldr_raw = analogRead(LDR_PIN);

ldr_percent = (float)ldr_raw / ADC_MAX * 100.0;

// --- DHT --- (read every sensor cycle as before)

float newHum = dht.readHumidity();

float newTemp = dht.readTemperature();

if (!isnan(newHum)) humidity = newHum;

if (!isnan(newTemp)) temperature = newTemp;

// Debug print

Serial.print("pH=");

Serial.print(ph_act, 3);

Serial.print(" | LDR=");

Serial.print(ldr_percent, 1);

Serial.print("% | T=");

Serial.print(isnan(temperature) ? 0 : temperature, 1);

Serial.print("°C | H=");

Serial.print(isnan(humidity) ? 0 : humidity, 1);

Serial.print("% | R1=");

Serial.print(relayState1 ? "ON" : "OFF");

Serial.print(" R2=");

Serial.print(relayState2 ? "ON" : "OFF");

Serial.print(" R3=");

Serial.println(relayState3 ? "ON" : "OFF");

}

void sendDataToServer() {

http.begin(client, sensor_endpoint);

http.addHeader("Content-Type", "application/json");

// Create JSON payload

DynamicJsonDocument doc(1024);

doc["ph"] = ph_act;

doc["pvolt"] = getLastPHVoltage();

doc["praw"] = (float)avgval / 6.0;

doc["ldr_raw"] = ldr_raw;

doc["ldr_pct"] = ldr_percent;

if (isnan(temperature)) {

doc["temp"] = nullptr;

} else {

doc["temp"] = temperature;

}

if (isnan(humidity)) {

doc["hum"] = nullptr;

} else {

doc["hum"] = humidity;

}

doc["relay1"] = relayState1;

doc["relay2"] = relayState2;

doc["relay3"] = relayState3;

String jsonString;

serializeJson(doc, jsonString);

int httpResponseCode = http.POST(jsonString);

if (httpResponseCode > 0) {

if (httpResponseCode == 200) {

Serial.println("Data sent successfully to HTTPS server");

} else {

Serial.print("HTTPS Error: ");

Serial.println(httpResponseCode);

}

} else {

Serial.print("HTTPS Connection error: ");

Serial.println(http.errorToString(httpResponseCode));

}

http.end();

}

void checkForCommands() {

http.begin(client, commands_endpoint);

int httpResponseCode = http.GET();

if (httpResponseCode == 200) {

String payload = http.getString();

DynamicJsonDocument doc(2048);

DeserializationError error = deserializeJson(doc, payload);

if (!error) {

JsonArray commands = doc["commands"];

for (JsonObject command : commands) {

processCommand(command);

}

if (commands.size() > 0) {

Serial.print("Processed ");

Serial.print(commands.size());

Serial.println(" commands");

}

} else {

Serial.print("JSON parsing error: ");

Serial.println(error.c_str());

}

} else if (httpResponseCode != 404) {

Serial.print("Command check error: ");

Serial.println(httpResponseCode);

}

http.end();

}

void processCommand(JsonObject command) {

String type = command["type"];

if (type == "relay") {

int relayId = command["relay_id"];

String state = command["state"];

bool on = (state == "on");

Serial.print("Relay command: R");

Serial.print(relayId);

Serial.print(" -> ");

Serial.println(state);

switch (relayId) {

case 1:

relayState1 = on;

applyRelayOutput(RELAY_PIN_1, relayState1);

Serial.print("Manual R1 command: ");

Serial.println(relayState1 ? "ON" : "OFF");

break;

case 2:

relayState2 = on;

applyRelayOutput(RELAY_PIN_2, relayState2);

break;

case 3:

relayState3 = on;

applyRelayOutput(RELAY_PIN_3, relayState3);

break;

}

}

}

void applyRelayOutput(int pin, bool state) {

// state = true: ON => active LOW => digitalWrite LOW

// state = false: OFF => digitalWrite HIGH

digitalWrite(pin, state ? LOW : HIGH);

}

float getLastPHVoltage() {

float avgRaw = (float)avgval / 6.0;

float volt = avgRaw * VREF / ADC_MAX;

return volt;

}